Tire Inspection and Recording

____Check air in all tires, record, and air up per manufactures specifications.

____Check tire tread depth, record, and replace if 6/32 or less on the front and 4/32 on less on the rear.

____Check date codes on tires, record, and replace if 9 years or older for class 61 and above. This includes 9x trailers. Change if 6 years or older on class 59 and down. Note mobile substation tires get changed every 5 years.

_____Check tires overall condition for solid sidewalls, cracks, dry rot, cuts, separation, bulges, irregular wear, and Items lodged in tires. Replace as necessary.

Front Axle

LF                                                    RF

_____ TP                                       _____TP

_____TD                                       _____TD

_____Year                                     _____Year

Rear axle

LR                                                  RR

_____ TP                                       _____TP

_____TD                                       _____TD

_____Year                                     ____Year